Image Blurring: Same Result in Space and in Frequency

Blurring by convolution (a spatial filter) and blurring by multiplying the Fourier transform by the filter's transform (a frequency-domain filter) give the same image. This follows from the convolution theorem.

1. What blurring is

A digital image is a grid of numbers, f[x, y]. Blurring replaces each pixel with a weighted average of the pixels around it:

g[x, y] = sum over all p, q of  h[p, q] * f[x - p, y - q]

This is called convolution, written f * h. The small grid of weights, h, is the kernel (also called a filter or mask). Two kernels are used here:

-  Gaussian (smooth and round). Sampled on a grid and rescaled so the weights add up to 1:

h[x, y] = (1 / (2 * pi * sigma^2)) * exp( -(x^2 + y^2) / (2 * sigma^2) )

-  Box / mean (uniform): every weight is 1 / k^2 over a k-by-k window.

A blur keeps smooth, slowly-changing parts of the picture and removes sharp edges and noise. Smooth parts are low frequencies; edges and noise are high frequencies. That is why a blur is called a low-pass filter, and why the picture looks soft.

2. The frequency domain

The 2-D discrete Fourier transform (DFT) rewrites an image as a sum of wave patterns. Each wave has a frequency (u, v), and F[u, v] is a complex number that says how much of that wave is in the image:

F[u, v] = sum over x, y of  f[x, y] * exp( -j * 2 * pi * (u*x/M + v*y/N) )

Here j is the imaginary unit (j^2 = -1), and M, N are the image width and height. Small u and v (near the origin) are low frequencies: smooth areas. Large u and v are high frequencies: edges and noise. The inverse DFT turns F back into f exactly.

3. The convolution theorem

This is the main result of the module. Let F = DFT of f and H = DFT of h. Then the DFT of a convolution equals the product of the two DFTs:

DFT of (f * h)  =  F * H      (that is,  F{ f * h } = F{f} * F{h} )

Proof (shown in 1-D; the 2-D case works the same, applied to both directions). Write the DFT of the convolution and rearrange:

F{f * h}[u]
  = sum over n of ( sum over m of  f[m] * h[n - m] ) * exp( -j * 2 * pi * u*n/N )
  = sum over m of  f[m] * sum over n of  h[n - m] * exp( -j * 2 * pi * u*n/N )
  ( put k = n - m, so n = k + m )
  = sum over m of  f[m] * sum over k of  h[k] * exp( -j * 2 * pi * u*(k + m)/N )
  = ( sum over m of  f[m] * exp( -j * 2 * pi * u*m/N ) )
      * ( sum over k of  h[k] * exp( -j * 2 * pi * u*k/N ) )
  = F[u] * H[u]

That proves it. What it means for blurring:

-  Blur in frequency: G[u, v] = F[u, v] * H[u, v], then g = inverse DFT of G.

-  H[u, v] (the DFT of the kernel) is the transfer function: it scales each frequency up or down. For a blur it is big at low frequencies and small at high frequencies (a low-pass shape).

-  One filter, two views: convolution in space and multiplication in frequency give the same image.

4. A Gaussian's transform is a Gaussian

The Gaussian kernel is special: its Fourier transform is another Gaussian.

exp( -x^2 / (2 * sigma^2) )  gives  sqrt(2 * pi) * sigma * exp( -2 * pi^2 * sigma^2 * u^2 )

So "convolve with a Gaussian of width sigma" is the same as "multiply by a Gaussian of width 1 / sigma" in frequency. A wide Gaussian in space (heavy blur) gives a narrow low-pass in frequency, and the other way around. A box kernel instead has a sinc-shaped transform, sinc(u) * sinc(v), which is why box blurs show faint ringing near edges.

5. Implementation and validation (evidence)

filtering.py computes the same blur two independent ways:

| Route | Operation | Code |
| --- | --- | --- |
| Spatial | g = f * h (slide the kernel over the image) | convolve2d_spatial (OpenCV), convolve2d_spatial_numpy (pure NumPy) |
| Frequency | G = F * H, then g = inverse DFT of G | convolve2d_frequency (NumPy FFT) |

One important detail: the DFT does circular convolution, but blurring with a zero border is linear convolution. So the frequency path zero-pads the image and kernel to (M + kh - 1) by (N + kw - 1) before the transform, then crops the middle M-by-N block. On the padded arrays, circular convolution equals linear convolution.

The experiment (experiment.py) blurs a test image both ways. The test image contains a gradient, a sharp square, a checkerboard, a sinusoid, a single bright dot, and random noise, so it has energy at every frequency. It uses Gaussian kernels (sigma = 0.5, 1.5, 3, 5) and box kernels (3x3, 11x11, 25x25).

Results (float64, saved in output/results.csv):

| configuration | kernel | max abs difference | RMSE | PSNR |
| --- | --- | --- | --- | --- |
| Gaussian sigma=0.5 | 5x5 | 2.8e-13 | 4.4e-14 | 315 dB |
| Gaussian sigma=1.5 | 11x11 | 3.6e-12 | 7.0e-13 | 291 dB |
| Gaussian sigma=3.0 | 19x19 | 3.5e-12 | 6.9e-13 | 291 dB |
| Gaussian sigma=5.0 | 31x31 | 2.9e-12 | 6.0e-13 | 293 dB |
| Box 3x3 | 3x3 | 4.0e-13 | 6.0e-14 | 313 dB |
| Box 11x11 | 11x11 | 3.3e-12 | 6.2e-13 | 292 dB |
| Box 25x25 | 25x25 | 2.2e-12 | 4.9e-13 | 294 dB |

The largest difference anywhere is about 1e-12, which is just floating-point round-off, and the PSNR is about 290 to 315 dB. When the results are rounded to 8-bit images, the saved files are byte-for-byte the same in six of seven cases; the one exception (box 3x3) differs by at most 1 grey level in 19 of 262144 pixels, at pixels that land exactly on a rounding boundary. The pure-NumPy convolution agrees with OpenCV to about 1e-13 on a test crop.

Conclusion. Convolution in space and multiplication in frequency produce the same blurred image, up to round-off, exactly as the convolution theorem predicts.

6. Running it

-  Web app: docker compose up, then open http://localhost:8000.

-  Evidence: python3 experiment.py (writes output/results.csv and plots).